Object Type:    difshell

Description:    Implementation of a concentration shell that can have

miscellaneous in/out flows (when coupled to other objects),

and one-dimensional diffusion between other difshells.

Author:         E. De Schutter BBF-UIA 4/94 - 3/99

------------------------------------------------------------------------------

ELEMENT PARAMETERS

DataStructure:  difshell_type [in src/concen/conc_struct.h]

Size:           152 bytes

Fields:         C               concentration (calculated by difshell) in mM

prev_C          concentration at previous time step

Ceq             equilibrium concentration in mM

D               diffusion constant in m^2/sec

val             ionic valence

leak            constant in/outflow in mM/sec

shape_mode      Set to one of the predefined global

variables SHELL=0, SLICE=SLAB=1, USERDEF=3.

SHELL: onion shell, for radial diffusion. Set len for

cylindrical shell (zero length for spherical shell), dia

(outer diameter of shell) and thick (thickness of shell);

vol, surf_up, and surf_down are computed on RESET.

SLICE (or SLAB): salami slices of cylinder, for axial

diffusion. Set dia (diameter of cylinder) and thick

(thickness of slab), len is not used; vol, surf_up, and

surf_down are computed on RESET.

USERDEF: len, dia, thick are not used. Set vol, surf_up

and surf_down.

len             shell length

dia             shell diameter

thick           shell thickness

vol             shell volume

surf_up         area of upper (outer) shell surface

surf_down       area of lower (inner) shell surface

------------------------------------------------------------------------------

SIMULATION PARAMETERS

Function:       DifShell [in src/concen/difshell.c]

Classes:        concentration segment

Actions:        RESTORE2  SAVE2  CHECK  RECALC  RESET  PROCESS  INIT

Messages:	INFLUX      I_Ca          (current flow in A; increases C)

OUTFLUX     I_Ca          (current flow in A; decreases C)

FINFLUX     I_Ca fraction (current flow, scaled by fraction)

FOUTFLUX    I_Ca fraction (current flow, scaled by fraction)

STOREINFLUX flux          (flux into store in moles/sec;

increases C)

STOREOUTFLUX flux         (flux out of store in moles/sec;

decreases C)

DIFF_DOWN   prev_C thick  (diffusion to inner shell, sent from

an outer shell)

DIFF_UP     prev_C thick  (diffusion to outer shell, sent from

an inner shell)

BUFFER      kBf kBb Bfree Bbound    (fields sent from a buffer,

e.g. a fixbuffer)

TAUPUMP     kP Ceq        (sent from a taupump)

EQTAUPUMP   kP            (sent from a taupump; uses Ceq from

the difshell)

MMPUMP      vmax Km       (sent from a mmpump)

HILLPUMP    vmax Kd Hill  (sent from a hillpump)

------------------------------------------------------------------------------

Notes:

The concentration of intracellular calcium close to the cell membrane surface

can have a significant influence on the conductance of calcium dependent

potassium channels and on mechanisms for synaptic plasticity.  Unless the

neural compartment is very small and the calcium is "well-mixed", the calcium

concentration may vary considerably within the compartment.  It may then be

necessary to model diffusion within the compartment by dividing it into a

number of shells.  The difshell object is used for modeling these shells and

diffusion between them.  It may also be used for modeling axial diffusion

between compartments.  Just as the length of a neural compartment should be

chosen to give small differences in membrane potential between adjacent

compartments, the thickness of diffusion shells should be chosen to give small

differences in concentration between adjacent shells.

A difshell is typically coupled to buffers and/or pumps, in order to provide

for multiple mechanisms for calcium removal.

The buffers (e.g, fixbuffer) interact with a difshell or concpool to model the

binding of calcium to buffer molecules and to calculate the concentrations of

free and bound ions using a kinetic scheme with forward and backward rate

constants for the binding and release of calcium.  The BUFFER message from a

buffer passes on the rate constants and the calculated free and bound

concentrations so that the difshell may calculate the resulting ion

concentration C in the shell.

The ionic pumps provide various mechanisms to remove ions from the shell.

Details of the fields that are passed to the difshell with messages from

the pumps are provided in the documentation for the various pump objects.

The various FLUX messages assume SI units for the Faraday constant, currents

and dimensions.  If other units are used, scale the val parameter to account

for this.

NOTE: When using difshell, difbuffer, or dif2buffer elements without hsolve

(or in chanmodes 0 or 1), it will be necessary to issue two reset commands

after setting up the simulation.

For a general description of buffered calcium diffusion see:

De Schutter E., and Smolen P., "Calcium dynamics in large neuronal

models", in Methods in neuronal modeling: From ions to networks

(2nd edition), C. Koch and I. Segev editors, pp. 211-250 (1998).  The

spinedemo simulation gives a detailed example of the use of the difshell,

taupump, and fixbuffer objects for modeling calcium diffusion in dendritic

spines.

Example:

/* based on spinedemo, for spine head initial shell (headshell0) */

create difshell headshell0

setfield headshell0 C 0.000020 Ceq 0.000020 D 6.0e-10 val 2.0  leak 0.0

setfield headshell0 shape_mode {SLAB} len 0.0 dia 0.5e-6 thick 0.1e-6

/* Ca fraction of NMDA current is influx into outer shell */

addmsg Mg_block headshell0 FINFLUX  Ik 0.128

/* attach fast immobile buffer */  headbuf0

create fixbuffer headbuf0

setfield headbuf0 Btot 0.2 kBf 5.0e5 kBb 500 // mM, 1/(mM*sec), 1/sec

addmsg headbuf0 headshell0 BUFFER kBf kBb Bfree Bbound

addmsg headshell0 headbuf0 CONCEN C

/* attach the pump */

create taupump headpump0

setfield headpump0 Ceq 0.000020

/* The pump rate is proportional to the surface-to-volume ratio

spine head initial shell is end of cylinder, so stv=1/thick */

setfield headpump0 T_C {thick/Pump_kP} // pump rate Pump_kP = 1.4e-5

addmsg headpump0 headshell0 TAUPUMP kP Ceq

/* set up diffusion messages, assuming the existence of headshell1 */

addmsg headshell0 headshell1 DIFF_DOWN prev_C  thick

addmsg headshell1 headshell0 DIFF_UP   prev_C thick

// similar statements for the other shells ...

See also:   taupump, fixbuffer, Scripts/examples/spinedemo